| Exp No: 12 | End-End Communication at Transport Layer - Echo Client Server and Chat |
| --- | --- |

AIM:

To implement (a) an echo client-server using TCP and UDP sockets and (b) a chat program using socket programming, for end-to-end communication at the transport layer.

ALGORITHM / PROCEDURE:

# (a1) TCP echo server and client

- Server: create a TCP socket with socket(AF_INET, SOCK_STREAM), bind() it to an IP address and port (5000), call listen() and wait in accept() for the client.

- Client: create a TCP socket and call connect() with the IP address and port of the server.

- The client sends a message using send(). The server reads it with recv(), displays it and sends the same data back using sendall().

- The client receives the echoed message and displays it. This is repeated for each message.

- Close the sockets using close().

# (a2) UDP echo server and client

- Server: create a UDP socket with SOCK_DGRAM and bind() it to port 6000. No listen() / accept() is needed because UDP is connectionless.

- Client: create a UDP socket and send the message to the server address with sendto().

- The server receives the datagram and the client address with recvfrom() and sends it back using sendto().

- The client receives the echo with recvfrom() and displays it. Close the sockets.

# (b) Chat program

- The chat server creates a TCP socket on port 7000 and waits for a client. The client connects to it.

- The client types a message and sends it. The server displays it, then the server user types the reply and sends it back.

- Messages are exchanged alternately until either side sends the word exit. Then both sockets are closed.

| Point | TCP | UDP |
| --- | --- | --- |
| Socket type | SOCK_STREAM | SOCK_DGRAM |
| Connection | Connection oriented (3-way handshake) | Connectionless |
| Reliability | Reliable, ordered delivery | No guarantee |
| Server calls | bind, listen, accept, recv, send | bind, recvfrom, sendto |
| Used in | HTTP, FTP, SMTP | DNS, DHCP, streaming |

DIAGRAM:

bind()

socket()

listen()

recv()

data

send()

echo of

data

close()

accept()

connect()

recv()

close()

send()

socket()

TCP

Server

TCP

Client

connection request (3

-

way

handshake)

Fig 1: Socket calls in TCP client-server communication

echo_tcp_client.py

import socket c = socket.socket(socket.AF_INET, socket.SOCK_STREAM) c.connect(("127.0.0.1", 5000)) for msg in ["Hello Server", "Computer Networks Lab", "bye"]:

bind()

recvfrom()

datagram

sendto()

echo of

datagram

socket()

close()

recvfrom()

sendto()

close()

socket()

UDP

Server

UDP

Client

Fig 2: Socket calls in UDP client-server communication

PROGRAM:

echo_tcp_server.py

import socket s = socket.socket(socket.AF_INET, socket.SOCK_STREAM) s.bind(("127.0.0.1", 5000)) s.listen(1) print("TCP Echo Server listening on port 5000 ...") conn, addr = s.accept() print("Connected by", addr) while True: data = conn.recv(1024) if not data: break print("Received :", data.decode()) conn.sendall(data) # echo back conn.close() s.close() print("Connection closed.")

echo_tcp_client.py (contd.)

c.sendall(msg.encode()) print("Sent :", msg) print("Echoed :", c.recv(1024).decode()) c.close()

echo_udp_server.py

import socket s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) s.bind(("127.0.0.1", 6000)) print("UDP Echo Server waiting on port 6000 ...") for _ in range(2): data, addr = s.recvfrom(1024) print("Received from", addr, ":", data.decode()) s.sendto(data, addr) s.close()

echo_udp_client.py

import socket c = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) for msg in ["Hello UDP", "Datagram test"]: c.sendto(msg.encode(), ("127.0.0.1", 6000)) data, _ = c.recvfrom(1024) print("Sent :", msg) print("Echoed :", data.decode()) c.close()

chat_server.py

import socket s = socket.socket() s.bind(("127.0.0.1", 7000)) s.listen(1) print("Chat server started. Waiting for client ...") conn, addr = s.accept() print("Client connected:", addr) while True: msg = conn.recv(1024).decode() if not msg or msg.lower() == "exit": break print("Client :", msg) reply = input("Server : ") conn.send(reply.encode()) if reply.lower() == "exit": break conn.close() s.close() print("Chat ended.")

chat_client.py

import socket c = socket.socket() c.connect(("127.0.0.1", 7000)) print("Connected to chat server. Type 'exit' to quit.") while True: msg = input("Client : ") c.send(msg.encode()) if msg.lower() == "exit": break reply = c.recv(1024).decode() if reply.lower() == "exit": break print("Server :", reply) c.close() print("Chat ended.")

OUTPUT:

# (a1) TCP echo

uUsSeErR @ UuUsSeErR - VirtualBox: ~ ( Terminal 1 - Server)

UuUSsEeRr @uUsSeErR -VirtualBox:~$ python3 echo_tcp_server.py TCP Echo Server listening on port 5000 ... Connected by ('127.0.0.1', 46248) Received : Hello Server Received : Computer Networks Lab Received : bye Connection closed.

UusSseEerRr @UuSsEeRr -VirtualBox: ~ (Terminal 2 - Client)

UuUSsEeRr @UuUsSeErR -VirtualBox:~$ python3 echo_tcp_client.py Sent : Hello Server Echoed : Hello Server Sent : Computer Networks Lab Echoed : Computer Networks Lab Sent : bye Echoed : bye

# (a2) UDP echo

UuUsSeErR @ uUsSseEerRr - VirtualBox: ~ ( Terminal 1 - Server)

uUsSseEerRr @UusSseEerRr -VirtualBox:~$ python3 echo_udp_server.py UDP Echo Server waiting on port 6000 ... Received from ('127.0.0.1', 53205) : Hello UDP Received from ('127.0.0.1', 53205) : Datagram test

uUsSseEerRr @UuUsSeErR -VirtualBox: ~ (Terminal 2 - Client)

uUsSeErR @UuUSsEeRr -VirtualBox:~$ python3 echo_udp_client.py Sent : Hello UDP Echoed : Hello UDP Sent : Datagram test Echoed : Datagram test

# (b) Chat program

UusSseEerRr @ uUsSeErR - VirtualBox: ~ ( Terminal 1 - Chat Server)

UusSseEerRr @UusSseEerRr -VirtualBox:~$ python3 chat_server.py Chat server started. Waiting for client ... Client connected: ('127.0.0.1', 43260) Client : Hi Server, this is uUsSeErR Server : Hello UusSseEerRr , how can I help? Client : Is the CN lab record ready? Server : Yes, the lab record is due on Friday. Client : Thanks, bye Server : exit Chat ended.

uUsSeErR @ UuSsEeRr - VirtualBox: ~ ( Terminal 2 - Chat Client)

UuUsSeErR @UuUsSeErR -VirtualBox:~$ python3 chat_client.py Connected to chat server. Type 'exit' to quit. Client : Hi Server, this is UuUsSeErR Server : Hello UuUsSeErR , how can I help? Client : Is the CN lab record ready? Server : Yes, the lab record is due on Friday. Client : Thanks, bye Chat ended.

RESULT:

Thus the echo client-server programs using TCP and UDP sockets and the chat program using socket programming were implemented and executed successfully.