| Exp No: 2 | Study of Network cables - straight-through and crossover cable |
| --- | --- |

AIM:

To study the different types of network cables and to make a straight-through cable and a cross-wired (crossover) cable using a crimping tool.

APPARATUS REQUIRED:

- Cat5e / Cat6 UTP cable (about 1 metre for each cable)

- RJ45 connectors (4 numbers)

- Crimping tool with built-in cutter and stripper

- LAN cable tester (main unit and remote unit)

ALGORITHM / PROCEDURE:

# Part A - Study of network cables

- Observe the sample twisted pair, coaxial and fibre optic cables available in the lab.

- Note the outer jacket marking (for example Cat5e, 24 AWG) and the number of pairs.

- Write down the use, speed and maximum length of each cable type in the table given below.

# Part B - Making the cables

- Cut the UTP cable to the required length (about 1 m) using the cutter.

- Remove about 2.5 cm of the outer jacket from both ends with the stripper. Take care not to cut the inner wires.

- Untwist the four pairs and straighten the eight wires with fingers.

- For the straight-through cable: arrange the wires in T568B order on both ends.

- For the crossover cable: arrange T568B order on one end and T568A order on the other end.

- Cut the wire tips even so that about 1.3 cm of the wires is left outside the jacket.

- Hold the RJ45 connector with the clip facing down, push the wires inside till they touch the end of the connector. The jacket should also go inside a little.

- Put the connector in the crimping tool and press firmly so that the pins cut into the wires.

- Repeat steps 2 to 8 for the other end of the cable.

- Insert both ends into the LAN tester and check the LED sequence. Types of network cables

| Cable | Connector | Max. length | Speed | Used for |
| --- | --- | --- | --- | --- |
| Twisted pair (UTP / STP) | RJ45 | 100 m | 10 Mbps to 10 Gbps | LAN, connecting PCs to switch |
| Coaxial (RG-6, RG-59) | BNC / F-type | 185 m to 500 m | 10 Mbps (old Ethernet) | Cable TV, CCTV, old LANs |
| Fibre optic (single / multi mode) | SC / LC / ST | 2 km (multi) up to 40 km+ (single) | 1 Gbps to 100 Gbps | Backbone, long distance links |

| UTP category | Max. speed | Bandwidth |
| --- | --- | --- |
| Cat5 | 100 Mbps | 100 MHz |
| UTP category | Max. speed | Bandwidth |
| Cat5e | 1 Gbps | 100 MHz |
| Cat6 | 1 Gbps (10 Gbps up to 55 m) | 250 MHz |
| Cat6a | 10 Gbps | 500 MHz |

| Pin | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T568A | W/G | G | W/O | B | W/B | O | W/Br | Br |
| T568B | W/O | O | W/G | B | W/B | G | W/Br | Br |

W = white stripe, G = green, O = orange, B = blue, Br = brown

| Cable | Wiring | Used to connect |
| --- | --- | --- |
| Straight-through | T568B to T568B | PC to Switch, PC to Hub, Router to Switch (unlike devices) |
| Crossover | T568B to T568A | PC to PC, Switch to Switch, Hub to Hub, PC to Router (like devices) |

DIAGRAM:

RJ45 - End A 	RJ45 R- JE4n5d-BEnd A 	RJ45 - End B

1

2

3

4

5

6

7

8

1

2

3

4

5

6

7

8

1

2

3

4

5

6

7

8

4

5

7

8

6

3

2

1

1

2

3

4

5

6

7

8

1

2

3

4

5

6

7

8

1

2

3

4

5

6

7

8

4

5

7

8

6

3

2

1

Fig 1: Straight-through cable (pin 1 to 1, 2 to 2 ...) 	Fig 2: Crossover cable (1 to 3, 2 to 6, 3 to 1, 6 to 2)

OUTPUT:

# Colour order of the wires at both ends

W/O

O

W/G

B

W/B

G

W/Br

Br

## Cable 1: Straight-through cable (T568B - T568B)

W/G

G

W/O

B

W/B

O

W/Br

Br

Cable 2: Crossover cable (T568B - T568A)

# LAN cable tester result

| MAIN | 1 1 |  | 2 2 | LAN TESTER - Cable 1 (Straight) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3 4 5 6 3 4 5 6 | 7 7 | 8 8 |
| REMOTE | 1 1 |  | 2 2 | 3 4 5 6 3 4 5 6 | 7 7 | 8 8 |

LEDs glow in the order 1-2-3-4-5-6-7-8 : cable is OK

LAN TESTER

-

Cable 2

)

Crossover

(

MAIN

1

2

3

4

5

6

7

8

REMOTE

3

6

1

4

5

2

7

8

LAN TESTER

-

Cable 2

)

Crossover

(

MAIN

1

2

3

4

5

6

7

8

REMOTE

3

6

1

4

5

2

7

8

## LEDs glow in the order 3-6-1-4-5-2-7-8 : crossover cable is OK

| Cable | Main unit LED | Remote unit LED | Status |
| --- | --- | --- | --- |
| Straight-through | 1 2 3 4 5 6 7 8 | 1 2 3 4 5 6 7 8 | Working |
| Crossover | 1 2 3 4 5 6 7 8 | 3 6 1 4 5 2 7 8 | Working |

RESULT:

Thus the different types of network cables were studied. A straight-through cable and a crossover cable were made using the crimping tool and both were tested successfully with the LAN tester.