| Exp No: 4 | Setup and configure a LAN using a Switch and Ethernet cables |
| --- | --- |

AIM:

To set up and configure a LAN using a switch and Ethernet cables, assign IP addresses to the hosts, check the connectivity using the ping command and share files and folders across the machines of the LAN.

ALGORITHM / PROCEDURE:

- Connect 4 host machines to the switch using straight-through UTP cables (RJ45). Check that the link LEDs of the switch glow.

- Assign a static IP address to each host: Windows, Control Panel > Network and Sharing Centre > Change adapter settings > Ethernet > Properties > IPv4 (or run ncpa.cpl).

- Use the addresses 192.168.10.11to  192.168.10.14with  subnet mask 255.255.255.0. Leave the default gateway empty because there is only one network.

- Open the Command Prompt and verify the settings with ipconfig.

- Check the connectivity between the machines with ping, for example from PC1 ping 192.168.10.12.

- On PC1 create a folder LabShare, right click > Properties > Sharing > Advanced Sharing > tick Share this folder and give Read/Write permission to Everyone.

- On PC2 open File Explorer and type \\192.168.10.11in  the address bar. The files of PC1 are displayed.

- The folder can also be mapped as a drive with net use Z: \\192.168.10.11\LabShareand  the shared files can be copied between the machines.

- Note down the output of each command.

| Host | IP address | Subnet mask | Switch port |
| --- | --- | --- | --- |
| PC1 | 192.168.10.11 | 255.255.255.0 | Fa0/1 |
| PC2 | 192.168.10.12 | 255.255.255.0 | Fa0/2 |
| PC3 | 192.168.10.13 | 255.255.255.0 | Fa0/3 |
| PC4 | 192.168.10.14 | 255.255.255.0 | Fa0/4 |

DIAGRAM:

Switch

PC

PC

PC

PC

SW

PC1

192.168.10.11

PC2

192.168.10.12

PC3

192.168.10.13

PC4

192.168.10.14

Fig 1: LAN with 4 hosts connected to a switch (star topology)

OUTPUT:

Command Prompt - PC1

C:\Users\UuUSsEeRr >ipconfig Windows IP Configuration Ethernet adapter Ethernet: Connection-specific DNS Suffix . : Link-local IPv6 Address . . . . . : fe80::6c1f:3a2e:91b4:d7c2%8 IPv4 Address. . . . . . . . . . . : 192.168.10.11 Subnet Mask . . . . . . . . . . . : 255.255.255.0 Default Gateway . . . . . . . . . :

Command Prompt - PC1

C:\Users\UuUsSeErR >ping 192.168.10.12 Pinging 192.168.10.12 with 32 bytes of data: Reply from 192.168.10.12: bytes=32 time=1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.10.12: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milliseconds: Minimum = 0ms, Maximum = 1ms, Average = 0ms C:\Users\UuUsSeErR >ping 192.168.10.13 Pinging 192.168.10.13 with 32 bytes of data: Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Reply from 192.168.10.13: bytes=32 time=1ms TTL=128 Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.10.13: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milliseconds: Minimum = 0ms, Maximum = 1ms, Average = 0ms C:\Users\UuUSsEeRr >ping 192.168.10.14 Pinging 192.168.10.14 with 32 bytes of data: Reply from 192.168.10.14: bytes=32 time<1ms TTL=128 Reply from 192.168.10.14: bytes=32 time<1ms TTL=128 Reply from 192.168.10.14: bytes=32 time<1ms TTL=128 Reply from 192.168.10.14: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.10.14: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Command Prompt - PC2

Volume in drive Z is LabShare Directory of Z: \ 07 - 10 - 2026 08:42 PM > < DIR . > dir Z: C: \ Users \ UuUsSeErR The command completed successfully . > net use Z: \ \ 192.168.10.11 \ LabShare C: \ Users \ UuUSsEeRr The command completed successfully . Disk LabShare Shared resources at \ \ 192.168.10.11 Share name Type Used as Comment > net view \ \ 192.168.10.11 C: \ Users \ uUsSseEerRr

Command Prompt - PC2 (contd.)

07-10-2026 08:42 PM <DIR> .. 07-10-2026 08:40 PM 14,336 CN_Lab_Notes.docx 07-10-2026 08:41 PM 212,480 Exp4_Topology.png 07-10-2026 08:42 PM 1,024 hosts_list.txt 3 File(s) 227,840 bytes 2 Dir(s) 48,213,327,872 bytes free C:\Users\UuUsSeErR >copy Z:\hosts_list.txt C:\Users\UuUsSeErR \Desktop 1 file(s) copied.

RESULT:

Thus a LAN was set up with 4 hosts connected to a switch using Ethernet cables, the IP addresses were assigned, the connectivity was verified using the ping command and files were shared and accessed across the machines of the LAN.