| Exp No: 9 | Implementation of SUBNETTING in CISCO Packet Tracer Simulator |
| --- | --- |

AIM:

To design multiple subnets with a suitable number of hosts, assign static IP addresses, connect the subnets through a router in Cisco Packet Tracer, and observe packet transmission with and without the router.

ALGORITHM / PROCEDURE:

- Take the network address 192.168.1.0/24 and decide the number of subnets needed (3 departments, so 4 subnets are created by borrowing 2 host bits).

- The new mask is 255.255.255.192 (/26). Each subnet has 64 addresses, so 62 usable hosts.

- Open Packet Tracer and place 1 router (2911), 3 switches (2960) and 6 PCs. Connect them with copper straight-through cables as in Fig 1.

- Assign a static IP address, subnet mask and default gateway to every PC under Desktop > IP Configuration.

- Configure the router interfaces: interface gig0/0, ip address 192.168.1.1

255.255.255.192, no shutdown

- Repeat step 5 for Gig0/1 and Gig0/2 with the gateway address of subnets B and C.

- From the PC command prompt use ping to test connectivity inside a subnet and across subnets.

- Remove the router (delete the links to the router) and ping across the subnets again to see the difference.

Fig 1: Network topology with three subnets joined by a router

PC1

PC0

Switch0

Switch1

Router0

(

Cisco

2911)

PC3

PC2

Gig0/1

PC5

PC4

Switch2

Gig0/0

Gig0/2

Subnet

A

Subnet

B

Subnet

C

Fig 2: Borrowing 2 bits of the host part for subnetting

Subnet bits (borrowed) 	Host bits (6)

| 1 | 1 | h | h | h | h | h | h |
| --- | --- | --- | --- | --- | --- | --- | --- |

192.168.1.0 /24 -> /26 (2 bits borrowed = 4 subnets, 2^6 - 2 = 62 hosts each)

Subnet design:

| Subnet | Network ID | Mask | Usable range | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- |
| A (Sales) | 192.168.1.0/26 | 255.255.255.192 | .1 - .62 | 192.168.1.63 | 192.168.1.1 |
| B (HR) | 192.168.1.64/26 | 255.255.255.192 | .65 - .126 | 192.168.1.127 | 192.168.1.65 |
| C (IT) | 192.168.1.128/26 | 255.255.255.192 | .129 - .190 | 192.168.1.191 | 192.168.1.129 |
| Unused | 192.168.1.192/26 | 255.255.255.192 | .193 - .254 | 192.168.1.255 | - |

IP address table:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router0 | Gig0/0 | 192.168.1.1 | 255.255.255.192 | - |
| Router0 | Gig0/1 | 192.168.1.65 | 255.255.255.192 | - |
| Router0 | Gig0/2 | 192.168.1.129 | 255.255.255.192 | - |
| PC0 | Fa0 | 192.168.1.2 | 255.255.255.192 | 192.168.1.1 |
| PC1 | Fa0 | 192.168.1.3 | 255.255.255.192 | 192.168.1.1 |
| PC2 | Fa0 | 192.168.1.66 | 255.255.255.192 | 192.168.1.65 |
| PC3 | Fa0 | 192.168.1.67 | 255.255.255.192 | 192.168.1.65 |
| PC4 | Fa0 | 192.168.1.130 | 255.255.255.192 | 192.168.1.129 |
| PC5 | Fa0 | 192.168.1.131 | 255.255.255.192 | 192.168.1.129 |

Router configuration:

Router>enable

Router#configure terminal

Router(config)#interface gigabitEthernet 0/0

Router(config-if)#ip address 192.168.1.1 255.255.255.192

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface gigabitEthernet 0/1

Router(config-if)#ip address 192.168.1.65 255.255.255.192

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface gigabitEthernet 0/2

Router(config-if)#ip address 192.168.1.129 255.255.255.192

Router(config-if)#no shutdown

Router(config-if)#end Router#write memory

OUTPUT:

Router0 CLI

-

show ip interface

brief

Router#show

ip

interface

brief

Interface

IP

-

Address

OK?

Method

Status

GigabitEthernet0/0

192.168.1.1

YES manual up

GigabitEthernet0/1

192.168.1.65

YES manual up

GigabitEthernet0/2

192.168.1.129

YES manual up

Protocol

up

up

up

PC0 Command Prompt

-

same

subnet

PC>

ping

192.168.1.3

Pinging

192.168.1.3

with

32

bytes

of

data:

Reply from 192.168.1.3: bytes=32 time=1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply

from

192.168.1.3:

bytes=32

time<1ms

TTL=128

Ping

statistics

for

192.168.1.3:

Pac

kets:

Sent

=

4

,

Received

=

4

,

Lost

=

0

(0

%

loss)

- When the subnets are connected via a router

PC0 Command Prompt

-

ping PC2 (Subnet B) through the

router

PC>

ping

192.168.1.66

Pinging 192.168.1.66 with 32 bytes of data:

Request timed out.

Reply from 192.168.1.66: bytes=32 time=3ms TTL=127

Reply from 192.168.1.66: bytes=32 time=2ms TTL=127

Reply

from

192.168.1.66:

bytes=32

time=2ms

TTL=127

Ping

statistics

for

192.168.1.66:

Packets:

Sent

=

4

,

Received

=

3

,

Lost

=

1

(25

%

loss)

PC0 Command Prompt

-

ping PC4 (Subnet C) through the

router

PC>

ping

192.168.1.130

Pinging

192.168.1.130

with

32

bytes

of

data:

Reply

from

192.168.1.130:

bytes=32

time=2ms

TTL=127

PC0 Command Prompt

-

ping PC4 (Subnet C) through the router

(

contd.

)

Reply from 192.168.1.130: bytes=32 time=1ms TTL=127

Reply from 192.168.1.130: bytes=32 time=1ms TTL=127

Reply

from

192.168.1.130:

bytes=32

time=2ms

TTL=127

Ping

statistics

for

192.168.1.130:

Packets:

Sent

=

4

,

Received

=

4

,

Lost

=

0

(0

%

loss)

- When the subnets are not connected (router removed)

PC0 Command Prompt

-

ping PC2 (Subnet B) without the

router

PC>

ping

192.168.1.66

Pinging 192.168.1.66 with 32 bytes of data:

Request timed out.

Request

timed

out.

Request

timed

out.

Request

timed

out.

Ping

statistics

for

192.168.1.66:

Packets:

Sent

=

,

4

Received

=

0

,

Lost

=

4

(100

%

loss)

PC0 Command Prompt

-

ping PC1 (same subnet) without the

router

PC>

ping

192.168.1.3

Pinging

192.168.1.3

with

32

bytes

of

data:

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply from 192.168.1.3: bytes=32 time<1ms TTL=128

Reply

from

192.168.1.3:

bytes=32

time<1ms

TTL=128

Ping

statistics

for

192.168.1.3:

Packets:

Sent

=

4

,

Received

=

4

,

Lost

=

0

(0

%

loss)

OBSERVATION:

| Case | Source -> Destination | Result |
| --- | --- | --- |
| Router connected | PC0 -> PC1 (same subnet) | Success, TTL = 128 |
| Router connected | PC0 -> PC2 / PC4 (other subnet) | Success, TTL = 127 (one router hop). First packet lost while ARP is resolved. |
| Router removed | PC0 -> PC1 (same subnet) | Success, switch forwards the frames |
| Router removed | PC0 -> PC2 / PC4 (other subnet) | Fails, 100% loss. No device can route between subnets. |

RESULT:

Thus subnetting of the 192.168.1.0/24 network into /26 subnets was implemented in Cisco Packet Tracer. Hosts in different subnets could communicate only when a router was connected between them, while hosts inside the same subnet could communicate even without the router.